Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#01)

Apply modern simulation tools ModelSim and

Logisim to implement basic processor components

Student Name: Syed Muhammad Shuja Abbas Naqvi 	Roll No:24I-6539	 Section: A

Date performed: 1st September, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task we Wrote Verilog codes for Basic gates using structural (Gate level) modeling and verifed result,basic gates included AND OR NOT

Code:

module logicgateAND(inputA,inputB,andout); input inputA; input inputB; output andout; and a1(andout,inputA,inputB); endmodule module logicgateOR(inputA,inputB,orout); input inputA; input inputB; output orout; or a1(orout,inputA,inputB); endmodule module logicgateNOT(inputA,notout); input inputA; output notout; not a1(notout,inputA); endmodule module testbench(); reg inputA; reg inputB; wire AND; logicgateAND uup(inputA,inputB,AND); wire OR; logicgateOR uut(inputB,inputB,OR); wire NOT; logicgateNOT abc(inputA,NOT); initial begin inputA=0; inputB=0; #50 inputA=0; inputB=1; #50 inputA=1; inputB=0; #50 inputA=1; inputB=1; end endmodule

Output:

Task 2:

Description:

In this task we Wrote Verilog codes for Basic gates using structural (data flow level) modeling and verifed result,basic gates included AND OR NOT

Code:

module logicgateAND(inputA,inputB,andout); input inputA; input inputB; output andout; assign andout=inputA&inputB; endmodule module logicgateOR(inputA,inputB,orout); input inputA; input inputB; output orout; assign orout=inputA+inputB; endmodule module logicgateNOT(inputA,notout); input inputA; output notout; assign notout=~inputA; endmodule module testbench(); reg inputA; reg inputB; wire AND; logicgateAND uup(inputA,inputB,AND); wire OR; logicgateOR uut(inputA,inputB,OR); wire NOT; logicgateNOT abc(inputA,NOT); initial begin inputA=0; inputB=0; #50 inputA=0; inputB=1; #50 inputA=1; inputB=0; #50 inputA=1; inputB=1; end endmodule

Output:

Task 3:

Description:

Here we tested the working of basic gates using logisim and got familiar with the interface and working of the program

Code:

NIL

Output:

Task 4:

Description:

Here we wrote Verilog code for circuit provided to us using gate level and data flow modeling and verified results using testbench.

Code:

module logicgateAND(inputA,inputB,andout); input inputA; input inputB; output andout; and(andout,inputA,inputB); endmodule module logicgateOR(inputA,inputB,orout); input inputA; input inputB; output orout; or(orout,inputA,inputB); endmodule module logicgateNOT(inputA,notout); input inputA; output notout; not(notout,inputA); endmodule module testbench(); reg inputA; reg inputB; wire notA; logicgateNOT G1(inputA,notA); wire notB; logicgateNOT G2(inputB,notB); wire AnotandB; logicgateAND G3(notA,inputB,AnotandB); wire BnotandA; logicgateAND G4(inputA,notB,BnotandA); wire finalout; logicgateOR G5(AnotandB,BnotandA,finalout); initial begin inputA=0; inputB=0; #50 inputA=0; inputB=1; #50 inputA=1; inputB=0; #50 inputA=1; inputB=1; end endmodule DATA FLOW module logicgateAND(inputA, inputB, andout); input inputA; input inputB; output andout; assign andout = inputA & inputB; endmodule module logicgateOR(inputA, inputB, orout); input inputA; input inputB; output orout; assign orout = inputA ¦ inputB; endmodule module logicgateNOT(inputA, notout); input inputA; output notout; assign notout = ~inputA; endmodule module testbench(); reg inputA; reg inputB; wire notA; logicgateNOT G1(inputA, notA); wire notB; logicgateNOT G2(inputB, notB); wire AnotandB; logicgateAND G3(notA, inputB, AnotandB); wire BnotandA; logicgateAND G4(inputA, notB, BnotandA); wire finalout; logicgateOR G5(AnotandB, BnotandA, finalout); initial begin inputA = 0; inputB = 0; #50 inputA = 0; inputB = 1; #50 inputA = 1; inputB = 0; #50 inputA = 1; inputB = 1; end endmodule

Output:

Task 5:

Description:

We recreated the circuit on logisim

Code:

NIL

Output:

Task 6:

Description:

Wrote Verilog code to find 1’s complement of four bits using gate level and data flow modeling. Verified results using testbench.

Code:

module compliment(inputA,outputA); input [3:0] inputA; output [3:0] outputA; not(outputA[0],inputA[0]); not(outputA[1],inputA[1]); not(outputA[2],inputA[2]); not(outputA[3],inputA[3]); endmodule module testbench(); reg [3:0] inputA; wire [3:0] outputA; compliment G1(inputA,outputA); initial begin inputA = 4'b0000; #50 inputA = 4'b0001; #50 inputA = 4'b1010; #50 inputA = 4'b1111; end endmodule DATA FLOW module compliment(inputA,outputA); input [3:0] inputA; output [3:0] outputA; assign outputA[0]=~inputA[0]; assign outputA[1]=~inputA[1]; assign outputA[2]=~inputA[2]; assign outputA[3]=~inputA[3]; endmodule module testbench(); reg [3:0] inputA; wire [3:0] outputA; compliment G1(inputA,outputA); initial begin inputA = 4'b0000; #50 inputA = 4'b0001; #50 inputA = 4'b1010; #50 inputA = 4'b1111; end endmodule

Output: