Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#6)

Use of Venus Simulator for RISC-V Procedure Execution

Student Name: Shuja Abbas 	Roll No:24i-6539	 Section: a

Date performed: 29th September, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

The procedure is not displaying the correct value because it changes the value of s0 from 2500 to 3 inside the procedure. Since s0 is a saved register, its original value should be preserved when a procedure uses it. Therefore, after returning, s0 becomes 3, and add t1, s0, a0 calculates 3 + 1003 = 1006 instead of 2500 + 1003 = 3503. To fix this, use a temporary register such as t0 instead of s0 for storing 3, so that s0 remains 2500. Since s0 is a saved register, a procedure should preserve its original value if it uses it. The easiest fix here is to use a temporary register f

Code:

.text main: li a0, 1 li s0, 2500 jal ra, procedure add t1, s0, a0 # t1 = 2500 + 1003 = 3503 li a0, 1 # display command mv a1, t1 # value to display ecall procedure: li t0, 3 li t1, 1000 add a0, t0, t1 # a0 = 1003 jalr zero, ra, 0

Output:

Task 2:

Description:

First, addi s0, zero, 5, addi t1, zero, 10, and addi t0, zero, 15 put the values 5, 10, and 15 into s0, t1, and t0. Then addi sp, sp, -12 reserves 12 bytes on the stack, since each register value takes 4 bytes. The three sw instructions save these values on the stack: s0 at 0(sp), t0 at 4(sp), and t1 at 8(sp). The lw instructions then retrieve these saved values into different registers: t4 gets 10, t2 gets 15, and t3 gets 5. Finally, addi sp, sp, 12 restores the stack pointer to its original position, effectively releasing the 12 bytes of stack space.

Code:

addi s0, zero, 5 addi t1, zero, 10 addi t0, zero,15 addi sp, sp, -12 sw s0, 0(sp) sw t0, 4(sp) sw t1, 8(sp) lw t4, 8(sp) lw t2, 4(sp) lw t3, 0(sp) addi sp, sp, 12

Output:

Task 3:

Description:

Here we wrote RISCV code for the C program which was provided

Code:

#int main(){ #x=EqSolver(a,b); #} #int EqSolver (int a, int b){ #return (a+b)(a-b); #} main: addi a0,zero,20 addi a1,zero,10 jal ra,EqSolver li a0,10 ecall EqSolver: add t0,a0,a1 sub t1,a0,a1 mul a1,t0,t1 ret

Output:

Task 4:

Description:

We wrote a RISCV assembly code to find the area and parameter of square , we did that by making to different functions and calling them in main

Code:

#Write RISC-V assembly code to find the area and perimeter of square. You need to #write two procedures (i.e. AreaFinder and PerimeterFinder ) and call both functions in #main module. main: li a1,10 li a2,10 jal ra,AreaFinder jal ra,PerimeterFinder li a0,10 ecall AreaFinder: mul a1,a1,a2 ret PerimeterFinder: slli a2,a2,2 ret

Output:

Task 5:

Description:

We converted the given C code into RISCV assembly code and ran the simulation

Code:

#Write RISC-V assembly code for given C code: #int main(){ #y=EqSolver(x); #} #int EqSolver (int x){ #return (x^3+5x^2+6x); #} main: li a1,10 jal ra,EqSolver li a0,10 ecall EqSolver: addi t0,a1,0 #t0=x mul t1,t0,t0 #t1=x^2 mul t2,t1,a1 #t2=x^3 li t3,5 li t4,6 mul t1,t3,t1 #t1=5*x^2 mul t0,t0,t4 #t0=6x add t5,t1,t0 add t5,t2,t5 add a1,t5,zero ret

Output:

Task 6:

Description:

Here we again converted C code into RISC-V assembly code

Code:

#int main(int n, int k){ # if (k == 1) #y=F1(n,k); # else if (n &gt; k) #y= F2(n); # else #y= F3(n); #} #int F1 (int x,int z){ #return (x 2 + z 2 ); #} #int F2 (int z){ #return (z 3 ); #} #int F3 (int w){ #return (w+5) main: li a0, 10 # n = 10 li a1, 20 # k = 20 li t0, 1 beq a1, t0, y1 blt a0, a1, y3 y2: jal ra, F2 j display y1: jal ra, F1 j display y3: jal ra, F3 display: addi a1, a0, 0 li a0, 1 ecall li a0, 10 ecall F1: mul t0, a0, a0 mul t1, a1, a1 add a0, t0, t1 ret F2: mul t0, a0, a0 mul a0, t0, a0 ret F3: addi a0, a0, 5 ret

Output:

Task 7:

Description:

We converted the given C code into RISCV assembly code and ran the simulation

Code:

#int main(int n, int k){ # if (k == 0) #y=fun1(n); # else if (n == k) #y= fun2(n); # else #y= fun3(n); #} #int fun1 (int x){ #return (x 2 ); #} #int fun2 (int z){ #return (z 3 ); #} #int fun3 (int w){ #return (w+1); #} main: li a0, 10 li a1, 0 beq a1, zero, y1 beq a0, a1, y2 jal ra, fun3 j exit y1: jal ra, fun1 j exit y2: jal ra, fun2 exit: addi a1, a0, 0 li a0, 1 ecall li a0, 10 ecall fun1: mul a0, a0, a0 ret fun2: mul t0, a0, a0 mul a0, t0, a0 ret fun3: addi a0, a0, 1 ret

Output:

Task 8:

Description:

// Write description of the task here 👇 (max. five to seven lines)

Code:

(a) main: li a0, 10 jal ra, fib j exit fib: beq a0, zero, base li t0, 0 li t1, 1 li t2, 2 loop: bgt t2, a0, done add t3, t0, t1 add t0, t1, zero add t1, t3, zero addi t2, t2, 1 j loop done: add a0, t1, zero ret base: addi a0, zero, 0 ret exit: addi a1, a0, 0 li a0, 1 ecall li a0, 10 ecall (b) main: li a0, 10 jal ra, fib j exit fib: beq a0, zero, base0 li t0, 1 beq a0, t0, base1 addi sp, sp, -12 sw ra, 8(sp) sw a0, 4(sp) addi a0, a0, -1 jal ra, fib sw a0, 0(sp) lw a0, 4(sp) addi a0, a0, -2 jal ra, fib lw t0, 0(sp) add a0, a0, t0 lw ra, 8(sp) addi sp, sp, 12 ret base0: li a0, 0 ret base1: li a0, 1 ret exit: addi a1, a0, 0 li a0, 1 ecall li a0, 10 ecall

Output: