Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#2)

Implementation of Combinational Circuits using Verilog Gates & DataFlow Level Modeling

Student Name: Syed Muhammad Shuja Abbas Naqvi Roll No: 24I-6539 Section: A

Date performed: 4th September, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task we Wrote Verilog Code for Half Adder and Full adder using Gate level and Dataflow level modeling. Simulated it by writing testbench and then made the circuit in logism.

Code:

GATE module halfadder(sum,carry,inputA,inputB); input inputA,inputB; output sum,carry; xor(sum,inputA,inputB); and(carry,inputA,inputB); endmodule module fulladder(SUM,cout,inputA,inputB,Cin); input inputA,inputB,Cin; output SUM,cout; wire sum1,c1,c2; halfadder h1(sum1,c1,inputA,inputB); halfadder h2(SUM,c2,sum1,Cin); or(cout,c1,c2); endmodule module testbench; reg inputA,inputB,Cin; wire sum,carry; wire SUM,cout; halfadder HA(sum,carry,inputA,inputB); fulladder FA(SUM,cout,inputA,inputB,Cin); initial begin inputA = 0; inputB = 0; Cin = 0; #50; inputA = 0; inputB = 0; Cin = 1; #50; inputA = 0; inputB = 1; Cin = 0; #50; inputA = 0; inputB = 1; Cin = 1; #50; inputA = 1; inputB = 0; Cin = 0; #50; inputA = 1; inputB = 0; Cin = 1; #50; inputA = 1; inputB = 1; Cin = 0; #50; inputA = 1; inputB = 1; Cin = 1; #50; end endmodule DATA FLOW module halfadder(sum,carry,inputA,inputB); input inputA,inputB; output sum,carry; xor(sum,inputA,inputB); and(carry,inputA,inputB); endmodule module fulladder(SUM,cout,inputA,inputB,Cin); input inputA,inputB,Cin; output SUM,cout; wire sum1,c1,c2; halfadder h1(sum1,c1,inputA,inputB); halfadder h2(SUM,c2,sum1,Cin); or(cout,c1,c2); endmodule module testbench; reg inputA,inputB,Cin; wire sum,carry; wire SUM,cout; halfadder HA(sum,carry,inputA,inputB); fulladder FA(SUM,cout,inputA,inputB,Cin); initial begin inputA = 0; inputB = 0; Cin = 0; #50; inputA = 0; inputB = 0; Cin = 1; #50; inputA = 0; inputB = 1; Cin = 0; #50; inputA = 0; inputB = 1; Cin = 1; #50; inputA = 1; inputB = 0; Cin = 0; #50; inputA = 1; inputB = 0; Cin = 1; #50; inputA = 1; inputB = 1; Cin = 0; #50; inputA = 1; inputB = 1; Cin = 1; #50; end endmodule

Output:

Task 2:

Description:

Here we Wrote Verilog code of 3x8 Decoder using Gate level and Dataflow level modeling simulated the results by writing testbench and made the circuit on logisim

Code:

module decoder3to8(Y,A,B,C); input A,B,C; output [7:0] Y; wire nA,nB,nC; not(nA,A); not(nB,B); not(nC,C); and(Y[0],nA,nB,nC); and(Y[1],nA,nB,C); and(Y[2],nA,B,nC); and(Y[3],nA,B,C); and(Y[4],A,nB,nC); and(Y[5],A,nB,C); and(Y[6],A,B,nC); and(Y[7],A,B,C); endmodule module testbench; reg A,B,C; wire [7:0] Y; decoder3to8 D(Y,A,B,C); initial begin A = 0; B = 0; C = 0; #50; A = 0; B = 0; C = 1; #50; A = 0; B = 1; C = 0; #50; A = 0; B = 1; C = 1; #50; A = 1; B = 0; C = 0; #50; A = 1; B = 0; C = 1; #50; A = 1; B = 1; C = 0; #50; A = 1; B = 1; C = 1; #50; end endmodule DATAFLOW module decoder3to8(Y,A,B,C); input A,B,C; output [7:0] Y; assign Y[0] = ~A & ~B & ~C; assign Y[1] = ~A & ~B & C; assign Y[2] = ~A & B & ~C; assign Y[3] = ~A & B & C; assign Y[4] = A & ~B & ~C; assign Y[5] = A & ~B & C; assign Y[6] = A & B & ~C; assign Y[7] = A & B & C; endmodule module testbench; reg A,B,C; wire [7:0] Y; decoder3to8 D(Y,A,B,C); initial begin A = 0; B = 0; C = 0; #50; A = 0; B = 0; C = 1; #50; A = 0; B = 1; C = 0; #50; A = 0; B = 1; C = 1; #50; A = 1; B = 0; C = 0; #50; A = 1; B = 0; C = 1; #50; A = 1; B = 1; C = 0; #50; A = 1; B = 1; C = 1; #50; end endmodule

Output:

Task 3:

Description:

In this task we Wrote Verilog code of 8x1 multiplexer using Gate level and Dataflow modeling. Simulate the results by writing testbench and then made the circuit on logism

Code:

module mux8to1(Y,I,S); input [7:0] I; input [2:0] S; output Y; wire nS2,nS1,nS0; wire w0,w1,w2,w3,w4,w5,w6,w7; not(nS2,S[2]); not(nS1,S[1]); not(nS0,S[0]); and(w0,I[0],nS2,nS1,nS0); and(w1,I[1],nS2,nS1,S[0]); and(w2,I[2],nS2,S[1],nS0); and(w3,I[3],nS2,S[1],S[0]); and(w4,I[4],S[2],nS1,nS0); and(w5,I[5],S[2],nS1,S[0]); and(w6,I[6],S[2],S[1],nS0); and(w7,I[7],S[2],S[1],S[0]); or(Y,w0,w1,w2,w3,w4,w5,w6,w7); endmodule module testbench; reg [7:0] I; reg [2:0] S; wire Y; mux8to1 MUX(Y,I,S); initial begin I = 8'b10101010; S = 3'b000; #50; S = 3'b001; #50; S = 3'b010; #50; S = 3'b011; #50; S = 3'b100; #50; S = 3'b101; #50; S = 3'b110; #50; S = 3'b111; #50; end Endmodule DATAFLOW module mux8to1(Y,I,S); input [7:0] I; input [2:0] S; output Y; assign Y = (~S[2] & ~S[1] & ~S[0] & I[0]) ¦ (~S[2] & ~S[1] & S[0] & I[1]) ¦ (~S[2] & S[1] & ~S[0] & I[2]) ¦ (~S[2] & S[1] & S[0] & I[3]) ¦ (S[2] & ~S[1] & ~S[0] & I[4]) ¦ (S[2] & ~S[1] & S[0] & I[5]) ¦ (S[2] & S[1] & ~S[0] & I[6]) ¦ (S[2] & S[1] & S[0] & I[7]); endmodule module testbench; reg [7:0] I; reg [2:0] S; wire Y; mux8to1 MUX(Y,I,S); initial begin I = 8'b10101010; S = 3'b000; #50; S = 3'b001; #50; S = 3'b010; #50; S = 3'b011; #50; S = 3'b100; #50; S = 3'b101; #50; S = 3'b110; #50; S = 3'b111; #50; end endmodule

Output: